PROJECT OTM

Warehouse Design — Analysis & Policy Pre-selection

Assa Abloy operation · DHL Supply Chain, Bemmel (NL_0032)

| Field | Detail |
| --- | --- |
| Deliverable | 1 — Analysis of data, pre-selection & justification of policies |
| Due date | 16-09-2026 |
| Data period analysed | Q2 2025 (01 Apr – 30 Jun) |
| Site constraints | ≤ 7,000 m² footprint · ≤ 12.2 m height |
| Scope | Receiving · Storage · Picking · Shipping (end-to-end) |
| Author | ________________________ |
| Version | DRAFT — analysis portion (design options / equipment to follow) |

This is a working draft covering the data-analysis portion of Deliverable 1. The design-choice pre-selection, policy substantiation and equipment selection (also part of Deliverable 1) are added as those work phases complete. All figures are measured from the provided Assa Abloy data; nothing is fabricated.

# 1. Introduction & scope

DHL Supply Chain previously ran warehouse operations for four clients in Tiel and recently relocated to a new site in Bemmel under significant time pressure. The Operations Manager considers the current (post-move) design sub-optimal. The objective of this project is to develop an alternative, more efficient and effective warehouse design for the Assa Abloy operation, based on a thorough analysis of the operational data.

The design covers four processes end-to-end — receiving, storage, picking and shipping — with all choices aligned across them. Facility architecture is out of scope; legal aspects (fire safety, compliance) are considered. The site envelope is a maximum footprint of 7,000 m² and a maximum height of 12.2 m.

# 2. Data sources & method

Two datasets from the Assa Abloy operation (warehouse NL_0032) form the primary source of truth:

| Dataset | Files | Grain | Volume |
| --- | --- | --- | --- |
| Storage / inventory | 3 snapshots (01-04/05/06-2025) | Inventory detail (DTLNUM) | 16,575–17,290 records |
| Outbound shipping & picking | 3 months (Apr–Jun 2025) | Order line | 39,069 lines · 8,781 orders |

Analysis is performed in Python (pandas). Every headline number is reproducible from the scripts in /analysis and cached in outputs/reports/phase1_metrics.json. Data-quality issues are flagged (Section 8), not silently corrected.

# 3. SKU profile

5,291 SKUs in stock across 11 product families.

3,115 SKUs have a case pack; 2,176 are each-only (≈41%) — confirming real each-pick demand.

Each size is small: median cube 0.0005 m³ (~500 cm³) — typical for locks, cylinders and fittings.

Units per pallet: median 1000 (mean skewed by high-count small parts).

Pallet weight: median 431 kg — realistic for a EURO pallet (see data-quality flag on sentinel maxima).

Pallet-type mix (SKU level): EURO 2,864, K3 2,040, XLONGPALLET 168 (long goods), plus K1/K2/K5/CONS.

Hazardous SKUs: 138 — require segregation/compliance consideration.

Fig 1. Pallet-weight distribution at SKU level (<p99).

# 4. Inventory & storage-position requirement

| Snapshot | Detail recs | Loads (positions) | Occupied locations | Active SKUs | Cube (m³) |
| --- | --- | --- | --- | --- | --- |
| 2025-04-01 | 16,575 | 10,176 | 7,715 | 5,164 | 3,209 |
| 2025-05-01 | 16,793 | 10,534 | 8,149 | 5,273 | 3,434 |
| 2025-06-01 | 17,290 | 11,015 | 8,512 | 5,291 | 3,679 |

Loads (≈ pallet positions) grew +8.2% in one quarter (10,176 → 11,015); cube +15%.

Peak concurrent loads = 11,015 → baseline storage-position requirement (before growth buffer and honeycombing losses).

Loads per SKU: mean 2.07, median 1, max 337; 72.7% of SKUs occupy a single load — a long slow-moving tail one pallet deep.

Eaches held roughly flat (~2.9M) while loads rose → stock is fragmenting into more, less-full locations: a current-design inefficiency to target.

Fig 2. Inventory footprint trend, Q2 2025 (loads and occupied locations).

# 5. Order & picking profile and lead time

| Metric | Mean | Median | p95 | Max |
| --- | --- | --- | --- | --- |
| Lines per order | 4.45 | 1 | 19 | 191 |
| Units per line | 34.77 | 5 | 150 | 12020 |
| Units per order | 154.7 | 8 | 575 | 20480 |

8,781 orders / 39,069 lines; 50.5% single-line orders and 23.6% single-unit lines — a small-order-heavy profile.

Pick-type split by lines: case/list 60.8%, each 16.0%, full-pallet 2.6%, unflagged/other 20.6% (see DQ flag).

Pick effort (lifts): case/list 279,669 vs each 30,165 vs pallet 27,801 → case picking is the dominant labour driver (~83% of lifts).

Delivery lead time (order received → dispatched): median 52.4 h (~2.2 days), p90 166.9 h.

Channel mix (order type): NLD 20,944, EDC 4,703, BEL 4,596, DIRECT 4,003, AMADE 2,216.

Geography: NLD 22,510, BEL 6,386, DEU 3,534, FRA 1,041, POL 911 — an NL hub serving Western Europe.

Fig 3. Order lines by pick type.

# 6. Demand-based ABC classification

ABC is recomputed from actual outbound demand (picked quantity), not assumed. The distribution is steeper than the classic 80/20 rule:

| Top X% of SKUs | % of picked volume |
| --- | --- |
| 5% of SKUs | 58.9% |
| 10% of SKUs | 74.5% |
| 20% of SKUs | 88.1% |
| 30% of SKUs | 94.0% |
| 50% of SKUs | 98.5% |

A/B/C by demand volume (A ≤ 80% cumulative, B ≤ 95%, C remainder):

| Class | SKUs | % SKUs | % volume | % pick-lines |
| --- | --- | --- | --- | --- |
| A | 565 | 13.0% | 80.0% | 41.8% |
| B | 858 | 19.7% | 15.0% | 30.0% |
| C | 2,931 | 67.3% | 5.0% | 28.3% |

~13% of SKUs drive 80% of volume, yet C-class is 67% of SKUs and still 28% of picks → forward-pick / fast-lane slotting is high-value.

Demand-ABC agrees with the WMS classification on only 48.6% of SKUs → strong evidence the current slotting is misaligned with real demand (re-slot).

4,354 SKUs shipped in Q2; 855 shipped were not in the latest stock snapshot (transient/cross-dock — to confirm).

Fig 4. Demand Pareto — cumulative picked quantity vs ranked SKUs.

# 7. Peak activity analysis

| Metric | Average / day | PEAK / day | Peak ÷ Avg | Peak date |
| --- | --- | --- | --- | --- |
| Order lines | 640 | 1171 | 1.83× | 2025-05-27 |
| Units | 22269 | 63469 | 2.85× | 2025-05-27 |
| Orders | 144 | 250 | 1.74× | 2025-04-29 |
| Shipments | 134 | 229 | 1.71× | 2025-04-29 |

Design to peak, not average. Units peak (2.85×) is far spikier than lines (1.83×), so sizing on lines alone understates pick/pack surges.

Weekly peak 3563 lines (week of 2025-06-01) vs ~2791 average.

Intraday peak around 08:00; Monday–Friday only (no weekend operations), weekdays fairly even.

Fig 5. Daily shipped order-lines with average and peak.

Fig 6. Average picks per hour of day.

# 8. Data-quality flags

Raised, not silently corrected:

PA WGT sentinel/impossible values — median 431 kg is fine, but p95 = 9999 kg and max = 60423 kg are impossible for one pallet (9,999 looks like a placeholder). To be cleaned before rack load-rating.

units_per_pallet extreme mean (15171 vs median 1000) — use robust statistics.

"Other" pick class = 20.6% of lines — no PALL/LIST/TROLLEY flag set; method needs a definition from DHL/WMS.

Stock vs demand mismatch — 855 shipped-not-stocked; a further set of stocked-not-shipped SKUs are dead-stock candidates. To confirm handling (cross-dock vs obsolete).

# 9. Headline design requirements (input to design options)

Storage: ~11,015 pallet positions today, growing ~+8.2%/quarter → target ≈ 12,000–13,000 positions incl. buffer, within the 12.2 m / 7,000 m² envelope (A008).

Long slow tail: 67.3% C-SKUs and 72.7% single-load SKUs → high-density/deep storage for the tail, fast forward-pick for A/B.

Picking is case-dominant (~83% of lifts) with meaningful each (16% of lines) and minor full-pallet (2.6%) → mixed pick methods; case-pick productivity is the key labour lever.

Re-slot to demand — only 48.6% ABC agreement today.

Capacity to peak — 1171 lines / 63469 units on the peak day; ~08:00 intraday peak.

# 10. Assumptions & open questions

Logged in data/assumptions.json. Items that materially affect the design:

| ID | Assumption / open question | Impact |
| --- | --- | --- |
| A001 | Max facility footprint | HIGH |
| A002 | Max clear/building height | HIGH |
| A003 | Analysis period is representative of normal operation | HIGH |
| A004 | One LODNUM (load) occupies one pallet position | HIGH |
| A005 | STAGED STATUS 'LATE' reflects staging vs the LATE SHIP window, i.e. a genuine on-time-staging miss | HIGH |
| A006 | No labour-productivity, MHE-fleet or cost data was provided | HIGH |
| A008 | The 7,000 m2 / 12.2 m envelope is the space available to DESIGN THE ASSA ABLOY operation within | HIGH |

Sources: project brief (client/university) and the Assa Abloy data files. External benchmarks (productivity, equipment specs, costs, safety codes) will be added with citations as later phases use them.