PROJECT OTM

Future-State Operational Design

Assa Abloy operation · DHL Supply Chain, Bemmel (NL_0032)

| Field | Detail |
| --- | --- |
| Deliverable | 2 — Proposed future state for the in-scope processes |
| Due date | 07-10-2026 |
| Concept | Option B — Hybrid density + velocity slotting + zone/batch picking |
| Envelope | ≤ 7,000 m² · ≤ 12.2 m |
| Team | Group 3 — Gleb, Rachel Selvarathnam, Arnau, Vidhi |
| Version | v3 (with Deliverable 3) — single-deep back-to-back racks, loose-container zone, machinery review |

Future-state design for the recommended Option B (Deliverable 1). Workload drivers are measured from the Assa Abloy Q2-2025 data; equipment data come from Jungheinrich; productivities and handling allowances are documented assumptions (Section 7) to be validated with DHL. Reproducible from analysis/*.py.

# Contents

(In Word: right-click → Update Field to build the table of contents.)

# 1. Recommended concept and what changed

Option B was selected in Deliverable 1. It combines single-deep pallet racking placed back-to-back (one pallet each side, every pallet reachable from an aisle), a module for odd items, a long-goods module with 4.5 m bays, bin shelving for small slow parts, and a fast-pick area (carton-flow / shelving) for the best-sellers, run with demand-based slotting and zone + batch picking. This version builds in the points agreed with the teacher and DHL:

| Topic | Before | Now (this version) |
| --- | --- | --- |
| Storage basis | 14,074 positions (11,015 loads + buffer) | ~9,500 positions: peak 8,512 occupied locations ÷ 0.90 |
| Pick method | DHL's historical pick flags (61% case) | Derived from the orders: 75% each / 24% case / 1% full-pallet lines |
| Inbound | Estimated from flow balance | Measured ~99 loads/day; 2 shipments/day from Assa Abloy factories, pallets + loose containers |
| Dock doors | 3 + 3 assumed | Calculated: 3 receiving (west, 1 container dock) + 5 shipping (east) |
| Equipment | Generic truck types (then ECE 225 for picking) | Jungheinrich ETV 216i ×2, EJE 120i ×3, ERE 225 ×2 + 6 pick trolleys; ECE 225 dropped (too long for the aisles, 2 pallets far too big for the orders) |
| Picking | One method for all orders | Split by order size: 84% small orders on trolleys (batch), 16% large orders picked onto a pallet |
| Aisles | Racks run to 1.5 m from the wall (dead ends) | Front + rear cross-aisles: every aisle open at both ends — no U-turns |
| Trailers | Leave whenever loaded | Departure plan by carrier (14 slots); 3 pick waves planned backwards |
| Loose containers | Unpacked at the dock door | Own 190 m² loose-container zone at door R1: unpack, sort per SKU, build pallets |
| Racking | Double-deep (telescopic forks) | Single-deep back-to-back: 8,960 positions incl. 320 long-goods, standard forks |
| Small parts | A pallet location each | 2,657 slow small SKUs in half-bins on 188 shelving bays next to the put-wall |
| 08:00 peak | Treated as customer demand | Caused by the order-release routine → release in waves |

The design follows the steps of the Week 3 design approach (Baker & Canessa, 2009, S017) and was iterated after each feedback round:

| Design step (W3) | Where it is done |
| --- | --- |
| Define requirements and objective | D1 §9 requirements; brief limits 7,000 m² / 12.2 m |
| Collect and analyse data | D1 §3–8 (SKUs, stock, orders, ABC, peaks); D2 §4.1 inbound |
| Process flows, handling, storage and throughput | D2 §2 processes; §4 docks, departures, order release |
| Equipment, storage and handling systems, policies | D2 §3 policies and COI slotting; §6.2–6.3 picking and machinery |
| Layout, zones and flows | D2 §5, incl. the U-shaped vs flow-through evaluation |
| Evaluate (cost, performance) | D2 §6 capacity and W3 KPIs; Deliverable 3 CAPEX/OPEX |
| Iterate | 14,074 → 9,500 positions; ECE 225 → trolleys + EJE 120i; front/rear cross-aisles; docks to side walls |

# 2. Future-state processes

Each process below states the steps, the system transaction, the physical movement, the equipment, and where the Deliverable-1 design choices are applied. The swimlane (Fig 1) gives the end-to-end overview; per-process flowcharts follow.

Fig 1. End-to-end future-state process — cross-functional swimlane.

## 2.1 Receiving & putaway

What arrives: 2 shipments a day straight from Assa Abloy factories (S004). Trailers from the European plants arrive palletised; sea containers from Asia (~41% of loads) arrive loose — cartons stacked on the container floor.

Palletised trailer (doors R2–R3): dock → unload with ERE 225 pallet truck → RF-scan against the ASN → quantity/quality check → register the load (LPN) → system-directed putaway with an ETV 216i reach truck → confirm.

Loose container (door R1, next to the loose-container zone): dock → a 2-person crew rolls the cartons out on an extendable roller conveyor, sorts them by SKU and builds pallets on the build spots → stretch-wrap, print LPN labels, scan against the ASN → the pallet goes to normal putaway. About 3 h per 40 ft container (A019, A026, Section 4.6).

Policy applied: demand-based directed putaway (A/B close to their fast-pick faces, C further away); pallets over 1 t on low levels; long goods (up to 2.05 m) in the long-goods module via its 3.2 m aisle.

Truck route: reach trucks use the left perimeter aisle and the cross-aisle, never the fast-pick area — Jungheinrich's 'clear traffic guidance' principle (S009).

Fig 2. Receiving & putaway flow.

## 2.2 Replenishment

Steps: fast-pick face below minimum → WMS replenishment task → ETV 216i takes the reserve pallet down the storage aisle → feeds the carton-flow lane from the rear → confirm (~24 pallets/day on average).

Policy applied: min/max replenishment; FIFO via FIF DATE where relevant. Pickers work the front of the lanes and trucks the rear, so people and reach trucks never share a face.

Fig 3. Replenishment flow.

## 2.3 Picking (zone + batch / wave)

The order decides the method (A022, A024). The typical order is only 7 litres (75% fit in one tote), while the biggest orders carry most of the volume. At release the WMS computes each order's volume and weight from the SKU data and sends it down one of three routes:

Small orders → pick trolley (84% of orders, 46% of lines): zone batch picking — a picker per zone fills a 6-tote trolley with the wave's lines up to its volume/weight limit (~31 lines per tour); the put-to-light wall sorts them per order.

Large orders → pick-to-pallet with an EJE 120i (16% of orders, 88% of volume): one order per pallet (median 234 L, 140 kg), dropped at packing.

Full-pallet lines (1% of lines) → ETV 216i straight from reserve to outbound.

Small slow parts (2,657 SKUs, ~93 lines/day) → bin shelving next to the put-wall: the item is picked into the order's tote and goes straight into the put-wall — no walk into the reserve aisles.

Where: A/B items in the fast-pick area (zone 1); other C items at the low levels of the reserve aisles (zones 2–3). Aisles are open at both ends, so pickers and pallet trucks walk straight through — no U-turns.

Why not the ECE 225: 3.7 m long with 2.4 m forks, it cannot turn in 3.0 m aisles, and its 2 pallets (~2,300 L) would need ~333 typical orders to fill one trip (Section 6.3).

Fig 4. Picking flow (zone + batch/wave).

## 2.4 Consolidation, packing & shipping

Steps: full trolleys of batch-picked totes → put-to-light wall sorts the items into orders → pack, print-and-apply → parcel cartons onto roll cages by carrier / large-order pallets wrapped → stage by carrier → live-load at the booked time → dispatch.

The put-to-light wall (Fig 5b): 24 × 5 = 120 slots, 10.8 m long, open on both sides; each slot holds one order tote. The putter scans an item, the slot of its order lights up with the quantity, the item goes in and the light is pressed. When an order is complete the light on the pack side goes on and the packer takes the tote out. Small slow parts are picked from the bin shelving directly behind the wall (Deliverable 3, §3.1).

Measured outbound: 15 trailers/day (p90 19), carrying on average only 1.2 pallets + 15 cartons; 73% of trailers carry no pallets at all (DHL Parcel routes).

Policy applied: carriers book collection time slots from a fixed departure plan (Section 4.5); goods wait in the outbound staging lanes, not in parked trailers; 5 shipping doors on the east wall (Section 4.4).

Fig 5. Consolidation, packing & shipping flow.

Fig 5b. Put-to-light wall: the packing area from above with the worker steps, and the front of the wall.

## 2.5 Returns (reverse logistics)

Steps: customer return arrives through the receiving doors → receive & scan (RMA) → inspect and grade in the Returns area → sellable: restock · reworkable: rework, then restock · otherwise scrap / return to vendor → update inventory and credit.

Policy applied: returns re-enter through the same demand-based putaway, so graded stock rejoins the reserve.

Fig 6. Returns / reverse-logistics flow.

# 3. Warehouse policies

| Policy | Decision | Why (data) |
| --- | --- | --- |
| Storage | Random within velocity zones; fixed fast-pick faces + random reserve; small slow parts in bins, not on pallets | 73% single-load tail; 48.6% ABC mismatch; 2,657 SKUs have ≤ 50 L of stock |
| Slotting | Demand-ABC; fast movers spread across zones, extreme SKUs duplicated; heavy low | 13% SKUs = 80% volume; A = 60% of picks (S003) |
| Zoning | Fast-pick (A/B) vs reserve; truck routes kept out of the pick zone | 75% of order lines are each-picks; safety (S009) |
| Batching | Small orders: zone batch picking, trolley filled to volume/weight; large: one order per pallet | median order 7 L; 88% of volume in large orders |
| Order release | 3 waves (06:00, 09:30, 12:45) planned backwards from trailer departures | 08:00 = 1.31× the mean hour (A023) |
| Routing | Largest gap in the reserve aisles; return route in the fast-pick area | shortest walk of 4 W3 policies tested (Section 6.2) |
| Putaway | System-directed, velocity-based to zone | align stock to demand |
| Replenishment | Min/max top-up from the rear of the lanes | keep faces filled at peak |
| Dock scheduling | Carrier time slots; stage, then live-load; 1 container dock | 15 trailers/day, 73% parcel-only; ~41% loose inbound |
| FIFO / FEFO | FIFO via FIF DATE where relevant | parts mostly not date-critical |

## 3.1 Peak-season slotting (congestion control)

At peak, congestion — not walk distance — becomes the dominant cost (Gadeyne, S003). Because the peak is spiky (1.83× lines / 2.85× units) and picks are concentrated, clustering all fast movers in one golden zone would jam a single area. We therefore adopt peak-aware slotting:

Picks are concentrated: the top 1% of SKUs (~43) do ~15.0% of all picks, and A-class SKUs ~60.0% of picks.

Spread fast movers across all pick zones (balanced on peak-day pick frequency): one zone would carry ~60.0% of picks; spread over 3 zones each carries ~20.0%.

Duplicate the extreme SKUs (~43 items) in different zones to allow parallel picking.

Levers in priority order (S003): slotting → order batching → wave-release timing → pick-path sequencing.

Validate by simulation at 150% / 200% / 250% of normal volume (Deliverable 3).

## 3.2 Slotting by Cube-per-Order Index (COI) and affinity

W3 key principle: the racking configuration should match each product's turnover, using the COI = storage space a SKU needs ÷ how often it is picked (S017). Low COI = little space, many picks → closest to packing.

| COI quintile | A SKUs | B SKUs | C SKUs |
| --- | --- | --- | --- |
| Q1 (best) | 161 | 172 | 121 |
| Q2 | 111 | 152 | 191 |
| Q3 | 83 | 124 | 247 |
| Q4 | 73 | 105 | 276 |
| Q5 (worst) | 56 | 70 | 328 |

COI confirms the ABC slotting: the best COI fifth of SKUs carries 55% of all picks, the best two fifths 72% — these fill the fast-pick area first.

COI corrects ABC in two places: 129 A items are bulky (poor COI) → kept on pallet faces at ground level near packing, not in carton-flow; 121 C items are tiny but picked often (best COI) → moved into the fast-pick shelving.

Affinity: in multi-line orders, 62% of item pairs share a product family vs 40% by chance → families are slotted together within each zone. Half of all orders are single-line, so affinity refines COI rather than replacing it.

# 4. Inbound, order release & dock doors

## 4.1 What comes in

No separate receiving file was provided, so receipts are measured from the storage snapshots: loads with an ADD DATE in the 14 days before each snapshot (A007). Loads already shipped before the snapshot are not visible, so these are lower bounds.

| Snapshot | Receiving days | Loads booked in | Loads / day | Busiest day | From Asia % |
| --- | --- | --- | --- | --- | --- |
| 2025-04-01 | 10 | 709 | 71 | 111 | 22% |
| 2025-05-01 | 9 | 974 | 108 | 167 | 54% |
| 2025-06-01 | 9 | 1,058 | 118 | 160 | 48% |

~99 loads are received per working day; the busiest day reached 167 (×1.7).

2 shipments a day, direct from Assa Abloy factories (teacher/DHL, S004) — in China, Germany, Romania, the Netherlands, Spain, Finland and others.

~41% of loads come from Asia and arrive loose in sea containers. The largest receipts (70–100 loads, all from China) take 1–3 days to book in — consistent with de-stuffing and palletising by hand.

## 4.2 Why picking peaks at 08:00

Fig 7. Orders arrive in the afternoon, are released at 16:00–17:00 and are picked the next morning — the 08:00 peak is the release backlog.

55% of orders arrive between 15:00 and 17:00, and ~43% are released (allocated) between 16:00 and 18:00.

Picking starts at 06:00 and peaks at 08:00 at 1.31× the average working hour: the whole afternoon backlog lands on the floor at the start of the shift (A023).

Future state: release work in 3 waves — W1 at 06:00, W2 at 09:30, W3 at 12:45 (W3 is the same-day wave for orders received before 12:00, Section 4.5). Levelling the load frees ~24% of peak-hour picker capacity without any change in customer service.

## 4.3 Inbound dock doors

Doors needed = the larger of (a) door-hours ÷ (receiving window 8 h × 85% utilisation) and (b) the number of arrivals that can be at the dock at the same time. A palletised trailer occupies a door ~1.3 h (unload with ERE 225 at ~1 min per pallet + 20 min dock-on and paperwork); a loose container ~3.3 h.

| Scenario | Door-hours / day | Doors by capacity | Arrivals at once | Doors needed |
| --- | --- | --- | --- | --- |
| Average day: 1 palletised trailer + 1 loose container | 4.6 | 1 | 2 | 2 |
| Peak day: palletised volume x1.7 + 1 loose container | 5.3 | 1 | 2 | 2 |
| Worst case: 2 loose containers (4 h de-stuff each) | 8.7 | 2 | 2 | 2 |
| Sensitivity: ~8 smaller trucks/day (WMS truck IDs) | 4.3 | 1 | 3 | 3 |

Result: 3 receiving doors on the west wall — R1 a container dock (leveller with the range for container chassis), R2–R3 for trailers. The two daily shipments need 2 doors so a 3-hour container never blocks a trailer; the third door covers the 'many smaller trucks' case, customer returns and breakdowns.

## 4.4 Outbound dock doors

Sized trailer by trailer from the 912 outbound trailers in Q2. Dock time per trailer = 15 min dock-on and paperwork + loading of its actual pallets and roll cages with an ERE 225 (mean 18 min, p90 22 min). Doors are sized on the busiest hour: p90 9 trailers × 18 min ÷ (60 min × 85%) = 3.1 → 4 doors, plus 1 spare.

Fig 8. Outbound doors needed today vs in the future state (busy day, p90).

| Measure | Value |
| --- | --- |
| Trailers per day (mean / p90 / max) | 15 / 19 / 22 |
| Load per trailer (mean) | 1.2 pallets + 15 cartons |
| Trailers with no pallets (parcel only) | 73% |
| Trailers dispatched in the busiest hour (p90 / max) | 9 / 12 |
| Today: trailers parked at the doors at once (p90) | 15 |
| Future without time slots (p90) | 8 |
| Future with carrier time slots | 3.1 → 5 doors incl. 1 spare |

Result: 5 shipping doors on the east wall, next to outbound staging and packing.

Carrier time slots are a condition of this design. WMS dispatch times are closed in batches (60% of afternoon dispatches are under 2 minutes apart); without booked slots up to 8 doors would be needed, and today's all-day parked trailers would need ~15.

Main carriers (trailers/day): Ex-works (customer pickup) 3.0, DHL DSV 2.4, DHL Parcel Netherlands 2.1, DHL Parcel NL DACH 1.4, DHL Parcel BE & LU 1.1, DHL4YOU 1.0.

## 4.5 When trailers leave — departure plan

Trailers get fixed departure slots, set close to each carrier's current median departure time (measured), and the picking waves are planned backwards from them: every wave finishes at least 45 minutes before the departure time of the trailers it feeds, leaving time to pack and stage.

Fig 9. Departure slots on the 5 shipping doors and the 3 pick waves that feed them (max 4 doors busy at once).

| Departure | Carrier | Trailers | Fed by wave |
| --- | --- | --- | --- |
| 10:30 | Ex-works pickup (AM) | 1 | W1 |
| 11:00 | DHL DSV run 1 | 1 | W1 |
| 13:30 | Ex-works pickup (PM) | 1 | W2 |
| 14:00 | DHL Parcel NL run 1 | 1 | W2 |
| 14:00 | Ex-works pickup (PM) | 1 | W2 |
| 14:30 | DHL DSV run 2 | 1 | W2 |
| 15:00 | Customer carriers | 2 | W2 |
| 16:00 | DHL Parcel NL DACH | 1 | W3 |
| 16:15 | DHL Parcel NL run 2 | 1 | W3 |
| 16:15 | DHL4YOU | 1 | W3 |
| 16:30 | DHL Connect & Return | 1 | W3 |
| 16:30 | DHL Parcel BE & LU | 1 | W3 |
| 16:45 | DHL Parcel EDC | 1 | W3 |

Waves: W1 06:00–09:30 (afternoon orders from the day before); W2 09:30–12:45 (rest of the backlog + morning orders); W3 12:45–15:15 (orders received before 12:00 (same-day parcel)).

Same-day service becomes possible: 32% of orders arrive before 12:00 and can go out on the afternoon parcel trailers via wave 3; later orders leave the next day.

Doors: at most 4 of the 5 doors are in use at once (25-minute slots), leaving one free for ex-works pickups that arrive late or early.

## 4.6 Loose containers — their own zone

After the last feedback meeting, loose containers get their own 190 m² zone directly behind door R1, between receiving and returns, so unpacking cartons does not share space with pallet receiving. The zone is sized from 568 Asian receipts in the stock data: a container brings a median of 20 pallets of 7 SKUs (p75: 34 pallets, 11 SKUs).

| Part of the zone | m² |
| --- | --- |
| Unload lane with extendable roller conveyor (12 x 2 m) | 24 |
| Pallet build spots (12 x 3 m2) | 36 |
| Finished-pallet buffer (35 x 1.9 m2) | 66 |
| Label / scan / stretch-wrap station | 16 |
| +30% walkways → needed | 185 |
| Planned in the layout | 190 |

| Who does the work | How | DHL labour | Pros | Cons |
| --- | --- | --- | --- | --- |
| In-house DHL crew | 2 DHL staff unpack each container in the loose-container zone at door R1 | 6 person-h per container | Full control, flexible timing | Heavy manual handling, training, peak days need 2 crews |
| On-site specialist contractor | Contractor crew unpacks and palletises in the same zone, paid per container | none | No own labour or peak staffing; specialist speed and safety; cost scales with containers | Per-container fee (no public price - quote needed); booking coordination |
| Port / CFS devanning | Container unpacked and palletised near the port; arrives as a palletised trailer | none | Simplest receiving - everything palletised | Extra transport and handling cost; ~1 day longer lead time; less control |

Is that enough for a big container? Unpacking spreads cartons over the floor while they are sorted. A median container needs ~134 m², the p90 container ~186 m² and the largest (89 pallets, 82 SKUs) ~458 m². 91% of containers fit the zone.

Big containers borrow door R2 (A044): receiving needs at most ~80 m² of its 262 m², because the two reach trucks put pallets away while the trailer is unloaded (peak ~17 pallets waiting). On a big-container day R2 is blocked and the north half of receiving (131 m²) becomes extra unpacking space: 321 m² in total, enough for 97% of containers.

Doors still fit: with R2 blocked, R1 (container) and R3 (trailers) remain — the base and peak scenarios need 2 doors. R2 is only blocked when the booked arrivals show no second trailer that day.

The few largest containers (more SKUs than build spots) are sorted in two passes: first the SKUs with the most cartons, then the rest from mixed pallets.

Decision rule: the zone is the same in both cases. A DHL crew costs about €128 per container (Deliverable 3); an on-site contractor is chosen only if its fixed price per container is lower (S024). Port devanning stays the fall-back if containers ever need to arrive palletised.

# 5. Layout & zoning (3-dimensional)

Zones are sized from the measured volumes and laid out as a straight-through flow: goods come in on the west side and leave on the east side. Building ≈ 96 m × 70 m.

| Zone | Area m² | % of building |
| --- | --- | --- |
| Receiving & inbound staging | 262 | 4% |
| Loose-container zone (unpack & palletise) | 190 | 3% |
| Offices & amenities | 250 | 4% |
| Returns / VAS | 180 | 3% |
| Single-deep pallet racks (A/B/C reserve) | 3,505 | 52% |
| Odd-item + long-goods modules | 728 | 11% |
| Fast-pick area (carton-flow / shelving) | 500 | 7% |
| Packing & consolidation | 450 | 7% |
| Outbound staging & shipping | 216 | 3% |
| Circulation / aisles | 450 | 7% |

Total floor area 6,731 m² = 96% of the 7,000 m² envelope.

Storage: 8,960 single-deep positions, counted on the drawing (11 back-to-back rack modules × 2 rows × 18 bays × 3 pallets × 8 levels) against a target of ~9,500 (peak 8,512 occupied locations ÷ 0.90). Top beam 10.0 m, within the ETV 216i's 10.7 m lift; one module (864 positions) holds odd items and one long-goods module holds 320 pallets of up to 2.05 m (4.5 m bays, two pallets each). Small slow parts leave the racks for bin shelving, which frees 1,005 locations: the peak need drops to ~7,507 locations (84% of the racks).

Working areas sized from measured volumes: inbound ~99 loads/day (peak 167); outbound 18 pallets + 225 cartons/day.

Docks: 3 receiving (west: R1 = container dock beside the loose-container zone, R2–R3 for trailers) and 5 shipping (east). A trailer door needs ~4.3 m of wall — the 9.9 m receiving and 9.5 m outbound columns are too narrow on the south wall, so the doors sit on the side walls. This also separates inbound and outbound truck traffic.

Aisles open at both ends: three 3.2 m cross-aisles (front, middle, rear) break up the racking, so no truck or picker ever needs a U-turn. Storage aisles are 3.0 m — wider than the ETV 216i needs (2.63–2.79 m, S006) and far wider than the EJE 120i (1.79–1.94 m, S010); the aisle in front of the long goods is 3.2 m (Section 5.2).

Fig 10. Detailed warehouse floor plan (rack-level, to scale) — racks, aisles, docks, dimensions.

Fig 11. Main flow on the floor plan: in (R1) → put away → refill → pick → pack → out (S2). Arrows follow the real truck and picker routes.

Fig 12. Option B in 3D (Blender): rack rows, fast-pick area, zones, 3 receiving doors (west) and 5 shipping doors (east).

Fig 13. Worker and truck movement (Blender, top-down): pickers with trolleys in zones 1–3, reach trucks ETV 216i via perimeter and cross-aisle, ERE 225 to the shipping doors.

## 5.1 Layout concept: U-shaped or flow-through?

W3 asks to produce a few basic concepts, evaluate them on travel distance and select the best. The two dock configurations of Huertas et al. (2007) were compared on the Option B layout, with class-based storage and the measured inbound mix:

| Criterion | Flow-through (chosen) | U-shaped |
| --- | --- | --- |
| Put-away travel (reach truck, per load, one way) | 65 m | 59 m (-9%) |
| Reach-truck time saved per day | - | ~11 min |
| Picking travel (all picks start/end at packing, east) | same | same |
| Dock doors | 8 (separate in/out) | ~6 if inbound always arrives before noon (shared) |
| Traffic and errors (Huertas et al.) | Inbound and outbound separated - less congestion, fewer mix-ups | Inbound, returns and outbound share one dock face |
| Fit to our profile (Huertas et al.) | Suits high volume, long/narrow buildings | Suits strong ABC (ours: 13% of SKUs = 80%) |

Verdict: Keep flow-through. U-shaped saves only ~9% put-away travel (~11 reach-truck minutes/day) because slow C items still bring 46% of inbound loads and picking travel is identical; its door saving depends on all inbound arriving before noon, which sea containers cannot guarantee. Flow-through keeps inbound, returns and outbound apart and fits the side-wall dock lengths.

## 5.2 Rack and aisle configuration — the W3 design questions

| Question (W3) | Our answer | Why |
| --- | --- | --- |
| How many aisles? | 10 storage aisles between 11 rack modules | 9 standard modules + 1 odd-items + 1 long-goods module |
| How long is each aisle? | ~56 m, split into two runs of ~26.5 m | middle cross-aisle halves the walk to any face |
| Cross-aisles — where, how many? | 3: front, middle, rear (3.2 m) | aisles open at both ends → largest-gap routing, no U-turns; costs ~6% rack length |
| How do pallets go into the racking? | EUR 800×1200 with the 800 mm side to the aisle, 3 per 2.7 m bay; one pallet deep per side, racks back-to-back; long pallets (≤ 2.05 m) 2 per 4.5 m bay | standard forks on the ETV 216i; long pallets forked on the long side (load centre 400 mm → full 1.6 t) |
| How wide is the aisle? | 3.0 m storage, 3.2 m for long goods and cross/perimeter | ETV 216i needs 2.63–2.79 m; with a 2.05 m load crosswise 2976 mm (VDI formula on S006 data) |
| 1-way or 2-way traffic? | storage aisles: one truck at a time; main aisles: trucks one-way (west up with put-aways, east down to packing) | pickers stay in their zones; fewer crossings |
| How high is each level? All the same? | 1.41 m pitch → 8 levels; taller loads on the top level | p90 load height 1.16 m + 0.25 m beam/clearance; top load ~1 m under the roof |
| Product characteristics? | heaviest item 90.0 kg per piece; pallets > 1 t low; longest real item ~2 m | mechanical handling only for heavy items; 115 loads recorded longer than 2.05 m (up to an '11 m' item of 0.5 kg) are data errors |

## 5.3 Legal aspects of pallet storage

Racking (EN 15512 / EN 15635, S015): racks designed to EN 15512; in use, a trained employee inspects visually every week and a competent expert at least every 12 months; damage is classed green/amber/red and every bay shows a load sign.

Fire safety: the top of the highest load stays ~1 m below the 12.2 m roof (sprinkler clearance assumed ≥ 0.5 m, A011 — to confirm against the sprinkler design).

Hazardous goods (rechecked, A028): 138 SKUs, 2,197 loads (20% of loads), ~385 t gross — counted per load, the same way in all three snapshots (1,727, 1,895, 2,197 loads). The customs codes show what they are (table below): 97% are electronic safes, locks, alarms, cameras and readers — most likely lithium cells built into the product (UN3481/UN3091, class 9). Those fall under PGS 37-2 (lithium energy carriers, from 333 kg; 1,000 kg for small cells, S029), not PGS 15. Only the lubricants (possibly aerosols) fall under PGS 15 (S016). A separate hazmat section is reserved; its fire separation follows PGS 37-2 once Assa Abloy confirms the UN numbers.

Manual handling: totes are limited to 15 kg (A024); items up to 90 kg each are only moved with pallet trucks or lift aids.

| Customs code | What it is | Likely hazard | Dutch rule | SKUs | Loads | Tonnes |
| --- | --- | --- | --- | --- | --- | --- |
| 8303 | Safes and strong-boxes (electronic locks) | lithium cells in equipment (UN3481 / UN3091, class 9) | PGS 37-2 | 41 | 1,603 | 315 |
| 8471 | Readers / data units | lithium cells in equipment (UN3481, class 9) | PGS 37-2 | 2 | 171 | 10 |
| 8301 | Locks and padlocks (electronic) | lithium cells in equipment (UN3481 / UN3091, class 9) | PGS 37-2 | 35 | 148 | 27 |
| 8531 | Alarm and signalling devices | lithium cells in equipment (UN3481 / UN3091, class 9) | PGS 37-2 | 25 | 141 | 17 |
| 8525 | Cameras | lithium cells in equipment (UN3481, class 9) | PGS 37-2 | 16 | 88 | 8 |
| 4819 | Cardboard boxes | unclear - packaging should not be hazardous; check the flag | check flag | 1 | 33 | 6 |
| 8302 | other | unknown | check | 8 | 10 | 0 |
| none | other | unknown | check | 2 | 7 | 1 |
| 3403 | Lubricants | possibly aerosols or flammable liquids (class 2 / 3) | PGS 15 | 5 | 6 | 0 |
| 8507 | Batteries (loose) | lithium batteries (UN3480 / UN3090, class 9) | PGS 37-2 | 1 | 5 | 0 |
| 8479 | other | unknown | check | 2 | 2 | 0 |

## 5.4 Are the outbound staging and packing areas big enough?

Outbound staging (216 m²): the goods of all afternoon trailers wait at the same time after wave 3. Measured per day (pallets × 1.9 m² + roll cages × 1.0 m², +30% lanes): median 34 m², p90 60 m², busiest day 100 m². 216 m² holds every day of Q2 with room for 2× the volume (stress test, D3).

Packing (450 m²) — what has to fit:

| Part | m² |
| --- | --- |
| Pack stations (4 x 10 m2: bench, scale, printer, working space) | 40 |
| Put-to-light wall (120 tote slots, 10.8 m, work strip both sides) | 39 |
| Large orders waiting to be packed (peak wave 13 pallets x 1.9 m2) | 25 |
| Stretch wrapper incl. safety zone | 12 |
| Trolleys and empty totes | 10 |
| Packaging material (one day of cartons, film, labels) | 40 |
| Small-parts bin shelving (188 bays, incl. aisles) | 207 |
| Total incl. 30% walkways (shelving already incl. aisles) | 423 |
| Planned | 450 |

# 6. Capacity plan — labour & equipment

## 6.1 Labour

Workload drivers are measured: inbound ~99 loads/day, outbound 18 pallets + 225 cartons/day, 483 each lines and 151 case lines/day, replenishment ~24 pallets/day. Picking hours come from the pick-time model in Section 6.2; other rates are benchmarks (A014). Unpacking loose containers is shown on its own line and not counted in the DHL FTE below, because it can be done by a DHL crew or a contractor (A026, costed in Deliverable 3).

| Process | Rate / h | Peak × | FTE avg | FTE peak |
| --- | --- | --- | --- | --- |
| Receiving - palletised (unload+check) | 25 | 1.69 | 0.31 | 0.52 |
| Loose-container unpacking (own zone, A026) | - | 2.0 | 0.80 | 1.60 |
| Putaway (reach truck) | 20 | 1.69 | 0.66 | 1.11 |
| Replenishment (reach truck) | 18 | 2.85 | 0.18 | 0.51 |
| Picking - small orders (trolley, batch) | - | 1.83 | 0.39 | 0.71 |
| Picking - large orders (EJE, to pallet) | - | 1.83 | 0.92 | 1.69 |
| Picking - small parts (bin shelving) | - | 1.83 | 0.25 | 0.46 |
| Full-pallet picking (reach truck) | 22 | 1.83 | 0.04 | 0.07 |
| Packing / consolidation | 45 | 1.83 | 1.88 | 3.44 |
| Shipping / loading | 30 | 2.85 | 0.08 | 0.22 |

Direct DHL labour ≈ 4.7 FTE on an average day and 8.7 FTE on the peak day (single shift, 7.5 productive hours; loose-container unpacking excluded). Cross-training covers the peak; supervision and admin come on top.

Fig 14. Labour requirement by process (average vs peak day).

## 6.2 Picking — how many people, how efficient

The picking method is matched to the demand profile — small, light orders with a few lines, plus a minority of pallet-sized orders — using the standard order-picking choices: picker-to-parts, low-level picking, batching and zoning (S014). Pick times are modelled on the Option B layout: walking (return route in the fast-pick area; in the reserve aisles the best of the four W3 routing policies, which is largest gap) + picking + setup, with a 10% delay allowance (A025).

Fig 15. Share of orders that fit each carrier: one tote, four totes, one pallet, two pallets (240×80).

| Segment | Orders / day | Lines / day | % orders | % lines | % volume |
| --- | --- | --- | --- | --- | --- |
| Large | 23 | 343 | 16% | 54% | 88% |
| Small | 120 | 291 | 84% | 46% | 12% |

| Small orders — zone | Lines / day | Lines / tour | Walk / tour (m) | Lines / h | Picker-h peak |
| --- | --- | --- | --- | --- | --- |
| Fast-pick area (A/B) | 188 | 31 | 123 | 99 | 3.5 |
| Reserve low levels (C) | 50 | 16 | 471 | 49 | 1.9 |

Large orders (EJE 120i): 23 orders/day, ~15 lines each (12 in the fast-pick area, 2 in the reserve), ~279 m and ~18 min per order → 44 lines/h.

Picking needs 1.6 FTE on an average day and 2.9 on the peak day. Trolley batching makes small-order picking about twice as fast as one-order-per-tote picking in the reserve zone.

How the volume per trip is ensured: each trolley batch is closed at 80% of its volume (240 L) or weight (250 kg), computed from SKU dimensions — so no trip is overloaded and none runs nearly empty.

| Item | Use | Needed | Planned | Why this number |
| --- | --- | --- | --- | --- |
| Pick trolley (3-tier, 6 totes) | Small orders - cluster picking in zones | 1 | 6 | 2 per picker (picking + at put-wall) + 2 spare |
| Totes 600x400 (~40 L) | One per small order (more if >40 L / >15 kg) | 36 | 54 | trolleys x 6 + 50% in circulation at packing |
| Jungheinrich EJE 120i walkie pallet truck | Large orders - pick-to-pallet | 3 | 3 | peak hours / (7.5 h x 80%); no spare once 2+ trucks (one breakdown still covers an average day) |

Tugger train (Jungheinrich EZS 130) — evaluated, not planned now. It fits zone picking (pickers stay in their zone and a train collects the trolleys), and the EZS 130 is built for small-parts tow trains (600 mm wide, tows 3 t, S012). But with batching there are only ~9 trolley trips a day: a 201 m loop every 20 minutes would keep a driver ~26% busy to save pickers ~6 minutes of walking, because both pick zones end next to packing. It becomes worthwhile if small-order volume roughly triples or packing moves away from the pick zones.

Routing policy (W3) — expected walk per tour in the reserve zone, by policy:

| Tour | Lines in reserve | S-shape | Return | Midpoint | Largest gap | Best |
| --- | --- | --- | --- | --- | --- | --- |
| Small-order trolley tour (reserve zone) | 16 | 572 m | 682 m | 488 m | 471 m | Largest gap |
| Large-order pallet tour (reserve part) | 2 | 168 m | 171 m | 162 m | 162 m | Largest gap |

Largest-gap routing is used in the reserve aisles (possible because the aisles are open at both ends); the fast-pick area is a single long face worked with a return route from packing.

Data-handling hardware (W3):

| Option | Fit with our profile | Decision |
| --- | --- | --- |
| Paper pick lists | 5,291 SKUs, many look-alike parts, half the orders single-line → error-prone, no live stock | Not used |
| RF handheld / ring scanner | scan-to-confirm location and item; works for receiving, put-away, trolleys and pallets | Used in all processes |
| Voice picking | hands-free, good for heavy case picking, but item check still needs a scan | Option for large-order picking |
| Pick-to-light | fastest per pick, but a light on thousands of faces is costly for ~640 lines/day | Not used in pick zones |
| Put-to-light wall | sorts batch-picked small orders (84% of orders) into orders at packing; 120 tote slots | Used at packing |

## 6.3 Machinery — Jungheinrich fleet

Truck models were chosen from Jungheinrich (as advised) to match the layout and loads, and sized with a cycle-time model built from their published speeds on the Option B routes (A021).

| Model | Task | Equip-h/day avg | Equip-h/day peak | Needed | Planned |
| --- | --- | --- | --- | --- | --- |
| ETV 216i | Putaway + replenishment + full-pallet picks | 4.9 | 9.2 | 2 | 2 |
| EJE 120i | Large orders: pick-to-pallet (one order per pallet) | 6.9 | 12.7 | 3 | 3 |
| ERE 225 | Unload trailers/containers, load outbound, dock moves | 2.2 | 4.6 | 1 | 2 |
| Pick trolley (3-tier, 6 totes) + totes | Small orders: zone batch picking, sorted at the put-wall | 2.9 | 5.3 | 1 | 6 |

ETV 216i reach truck (S006): 1,600 kg, lift 10.7 m, working aisle 2.63–2.79 m, Li-ion. 2 cover the peak day and one is enough on an average day, so a breakdown never stops putaway (no spare needed; the lease includes repair). The racks are single-deep, so the truck uses standard forks — no telescopic attachment and no loss of capacity at height.

EJE 120i walkie pallet truck (S010): 2,000 kg, only 1.58 m long and 0.72 m wide — picks large orders onto a pallet and moves through the aisles without turning problems.

Pick trolleys (S011): 3-tier, 1,000 × 600 mm, 250 kg, 6 totes — for the small orders.

ERE 225 rider pallet truck (S008): 2,500 kg, built for loading and unloading trailers and containers; one per dock side (west and east).

Charging: Li-ion opportunity charging — no battery room is needed; chargers sit at the P&D points.

Plus: RF / voice terminals per picker, print-and-apply at packing, roll cages for parcel staging.

Why each machine, and how it is used:

| Machine | Process | Why (data) | How it is used | Units |
| --- | --- | --- | --- | --- |
| ETV 216i reach truck | Putaway, replenishment, full-pallet picks | Pallets stored up to the 8th level (top beam 10.0 m) - needs 10.7 m lift; single-deep racks, so standard forks; 11,012 of 11,015 loads ≤ 1.6 t | Perimeter + cross-aisles; ~25 putaways/h; feeds fast-pick lanes from the rear | 2 |
| EJE 120i pallet truck | Large orders: pick-to-pallet | 16% of orders (23/day) are pallet-sized: median 234 L, 140 kg (p90 671 kg) - too big/heavy for a trolley | One order per pallet; ~15 lines in ~18 min; drops the pallet at packing | 3 |
| Pick trolley + totes | Small orders: zone batch picking | 84% of orders are small (median order 7 L; 75% fit in one tote) | WMS fills each trolley to its volume/weight limit (~31 lines per tour); put-to-light wall sorts per order | 6 |
| ERE 225 rider pallet truck | Unloading and loading trailers | ~99 inbound loads/day in the morning + outbound pallets in the afternoon; rider = less walking, 2.5 t | One per dock side (west receiving, east shipping) | 2 |
| EZS 130 tugger train | Transport picked trolleys to packing (option) | Fits zone picking, but only ~9 trolley trips/day; a driver would be 26% busy to save ~6 min walking | Not planned now; adopt if small-order volume ~triples or packing moves away from the pick zones | 0 |

Truck dimensions and turning — can every machine move where it has to?

| Machine | Size (L × W) | Turning / aisle needed | Where it drives | Result |
| --- | --- | --- | --- | --- |
| ETV 216i reach truck | 2,419 x 1,270 mm | Wa 1,665 mm; aisle 2,631-2,785 mm | Perimeter + cross-aisles; storage aisles | OK - turns 90° in the 3.2 m cross-aisles; with a 2.05 m long load it needs 2976 mm -> long goods only via 3.2 m aisles |
| EJE 120i pallet truck | 1,575 x 720 mm | Wa 1,414-1,464 mm; aisle 1,787-1,936 mm | Fast-pick area, storage aisles, cross-aisles | OK - aisles open at both ends, so no U-turn needed |
| Pick trolley (manual) | 1,000 x 600 mm | pushed by hand | Everywhere in the pick zones | OK |
| ERE 225 rider pallet truck | rider truck, 1,000 mm forks | dock work only | Dock strips and staging lanes (west + east) | OK - no aisle driving |
| EZS 130 tugger (option) | 1,315 x 600 mm + carts | Wa ~1,080 mm (tractor) | Loop on perimeter + cross-aisles only | OK for a short train; not planned (low volume) |
| ECE 225 order picker (dropped) | 3,670 x 810 mm, 2,400 mm forks | too long for 3.0 m aisles | - | Rejected - cannot turn or U-turn in the aisles; 2 pallets far too big |

| Move | Truck | Avg one-way (m) | Cycle (min) | Moves / h |
| --- | --- | --- | --- | --- |
| Putaway (receiving -> reserve) | ETV 216i | 78 | 2.38 | 25 |
| Replenishment (reserve -> fast-pick rear) | ETV 216i | 28 | 1.83 | 33 |
| Full-pallet pick (reserve -> outbound) | ETV 216i | 78 | 2.37 | 25 |
| Unload pallet (trailer -> receiving lane) | ERE 225 | 19 | 0.99 | 61 |
| Load pallet (staging -> trailer) | ERE 225 | 19 | 1.24 | 48 |

Fit checks — layout and loads against the truck data:

| Check | Requirement | Result |
| --- | --- | --- |
| Working aisle (reach aisle 3.0 m) | ETV 216i Ast 2,631-2,785 mm | OK - 0.2-0.4 m margin |
| Top beam 10.02 m (8 levels x 1.41 m) | Max lift 10.7 m | OK - 0.68 m spare |
| Clear height 12.2 m | Extended mast 11.7 m at max lift | OK - 0.5 m clearance |
| Pallet weight p95 875 kg / p99 1344 kg | 1,600 kg @ 600 mm | OK for 99.4% of SKUs; >1 t pallets slotted on low levels |
| Actual load weight in stock (qty x unit weight): 10 loads > 1 t, 3 > 1.6 t | 1,600 kg @ 600 mm | OK - the few loads > 1.6 t go to floor positions, moved with the ERE 225 (2.5 t) |
| Single-deep (back-to-back) racks | every pallet reachable from an aisle | OK - standard forks; no telescopic attachment, no residual-capacity loss |
| Long pallets: 140 loads 1.3-2.05 m (+115 recorded longer = data errors, capped at 2.05 m) | aisle >= Ast 2976 mm with a 2.05 m load crosswise | OK - long-goods module with 4.5 m bays (2 pallets each) behind a 3.2 m aisle; forks under the long side (load centre 400 mm, full 1.6 t) |
| Tall loads: 940 loads 1.16-1.68 m, 42 > 1.68 m | 1.41 m level pitch | OK - top level (~1.7 m free under the roof clearance) has 1,120 positions; > 1.68 m on the floor |
| Large orders up to p90 671 kg | EJE 120i 2,000 kg | OK - picked onto one pallet |
| Small orders (median 7 L) | Trolley 6 totes, 250 kg | OK - ~31 lines per tour |
| Charging | Li-ion opportunity charging | No battery room; chargers at P&D points |

## 6.4 Expected performance on the W3 KPIs

W3 names the KPIs that layout, storage and picking design drive. Expected values for Option B (Deliverable 3 adds cost and the full KPI set):

| KPI (W3) | Expected | Basis |
| --- | --- | --- |
| Dock-to-stock time | ~1.5 h per palletised trailer; ~3.8 h per loose container | unload ~1 min/pallet (ERE 225) while 2 reach trucks put away in parallel; container unpacked in its own zone at R1 |
| Put-away cycle time | 2.4 min per pallet (~25 per truck-hour) | ETV 216i cycle model on the Option B routes (S006) |
| Pick rate | ~59 lines per picker-hour overall (fast-pick 99, reserve 49, large orders 44) | pick-time model, largest-gap routing, trolley batching |
| On-time shipment rate | target ≥ 98% staged before the booked departure | waves end 45 min before each slot; today 50.1% of lines are staged 'LATE' (A005, definition to confirm) |

# 7. Assumptions & open questions

Full register in data/assumptions.json; sources in documentation/sources.md. High-impact items:

| ID | Assumption / open question | Impact |
| --- | --- | --- |
| A001 | Max facility footprint | HIGH |
| A002 | Max clear/building height | HIGH |
| A003 | Analysis period is representative of normal operation | HIGH |
| A004 | Storage requirement is based on OCCUPIED LOCATIONS (pallet positions), not loads | HIGH |
| A005 | STAGED STATUS 'LATE' reflects staging vs the LATE SHIP window, i.e. a genuine on-time-staging miss | HIGH |
| A006 | No labour-productivity, MHE-fleet or cost data was provided | HIGH |
| A008 | The 7,000 m2 / 12.2 m envelope is the space available to DESIGN THE ASSA ABLOY operation within | HIGH |
| A010 | Design pallet-position target | HIGH |
| A013 | Functional (non-storage) areas sized from actual order volume | HIGH |
| A014 | Labour productivity benchmarks (units/person-hour) | HIGH |
| A017 | Peak-aware slotting: spread fast movers, duplicate extreme SKUs, balance picks/aisle on peak forecast | HIGH |
| A018 | Inbound arrives as 2 shipments per day, direct from Assa Abloy factories | HIGH |
| A019 | Part of inbound arrives LOOSE (floor-loaded sea containers) and is de-stuffed and palletised by hand | HIGH |
| A020 | Dock-door planning parameters | HIGH |
| A022 | Pick type is derived from what the client ordered, not from DHL's historical pick flags | HIGH |
| A024 | Picking is split by ORDER SIZE computed from SKU each-dimensions and weights | HIGH |
| A025 | Pick-time model parameters | HIGH |
| A028 | Hazardous SKUs: hazard class unknown - PGS 15 (packaged hazardous substances) or PGS 37 (lithium batteries) to be confirmed | HIGH |
| A030 | Labour cost per paid hour | HIGH |
| A033 | Racking prices | HIGH |
| A042 | Small parts move from pallet racks to bin shelving | HIGH |

To confirm with DHL: labour productivity and pick-time standards (A006/A014/A025); the 7,000 m² scope (A008); container share and contractor quotes (A019/A026); whether carriers accept the departure slots (A020/A027).

To confirm with Jungheinrich: a lease quote for 2 ETV 216i, 3 EJE 120i and 2 ERE 225 (A038).

To confirm with Assa Abloy: the hazard class of the 138 flagged SKUs, which decides the hazmat section (A028).