PROJECT OTM

Future-State Operational Design

Assa Abloy operation · DHL Supply Chain, Bemmel (NL_0032)

| Field | Detail |
| --- | --- |
| Deliverable | 2 — Proposed future state for the in-scope processes |
| Due date | 07-10-2026 |
| Concept | Option B — Hybrid density + velocity slotting + zone/batch picking |
| Envelope | ≤ 7,000 m² · ≤ 12.2 m |
| Author | ________________________ |
| Version | DRAFT — processes, flowcharts, layout (3D), capacity plan |

Future-state design for the recommended Option B (Deliverable 1). Workload drivers are measured from the Assa Abloy Q2-2025 data; productivities are benchmarked assumptions (Section 6) to be validated with DHL. Reproducible from analysis/*.py.

# 1. Recommended concept (recap)

Option B was selected via the Deliverable-1 decision matrix (robust across base and cost-driven weightings). It combines double-deep reserve racking for A/B stock, selective + cantilever racking for irregular and long goods, and a forward-pick module (carton-flow / shelving) for the case- and each-heavy demand, operated with velocity-based slotting and zone + batch/wave picking.

# 2. Future-state processes

Each in-scope process below states the steps, the system transaction, the physical movement, the equipment, and where the Deliverable-1 design choices are applied. Flowcharts follow.

## 2.1 Receiving & putaway

Steps: trailer arrival → unload & RF-scan (ASN check) → quality/qty check → register stock (LODNUM) → system-directed putaway → reach-truck move → confirm.

Policy applied: velocity-based directed putaway (A/B to double-deep reserve near pick faces; C to deep/upper); cantilever for XLONG goods (43 SKUs >240 cm).

Fig 1. Receiving & putaway flow.

## 2.2 Replenishment

Steps: forward face below min → WMS replen task → pull reserve pallet → top-up carton-flow/pick face → confirm.

Policy applied: min/max replenishment; FIFO via FIF DATE where relevant.

Fig 2. Replenishment flow.

## 2.3 Picking (zone + batch / wave)

Steps: orders allocated → wave release by carrier cut-off → route by profile → full-pallet / case (batched) / each pick → to consolidation.

Policy applied: batch the 50% single-line orders; A-movers in a golden-zone forward pick; voice/RF direction; zone picking across areas.

Fig 3. Picking flow (zone + batch/wave).

## 2.4 Consolidation, packing & shipping

Steps: picked totes/pallets → put-to-light sort by order → pack & print-and-apply → parcel vs pallet → stage by carrier → load & dispatch.

Policy applied: put-to-light consolidation of multi-zone picks; pack sized to peak (labour bottleneck); parcel-heavy outbound (~18 palletised shipments/day).

Fig 4. Consolidation, packing & shipping flow.

# 3. Warehouse policies

| Policy | Decision | Why (data) |
| --- | --- | --- |
| Storage | Hybrid: random within velocity zones; fixed forward-pick faces + random reserve | 73% single-load tail; 48.6% ABC mismatch |
| Slotting | Velocity/demand-ABC (hot/med/cold); A in golden zone; heavy low | 13% of SKUs = 80% of volume |
| Zoning | Forward-pick vs reserve; pick zones by area | case-dominant picking (83% of lifts) |
| Batching | Batch single-line/single-unit orders; wave by carrier cut-off | 50% single-line, 24% single-unit |
| Routing | S-shape / return within aisles | reduce travel in reserve |
| Putaway | System-directed, velocity-based to zone | align stock to demand |
| Replenishment | Min/max forward-pick top-up | keep pick faces filled at peak |
| FIFO/FEFO | FIFO via FIF DATE where relevant | stock age present; most parts not date-critical (confirm) |

# 4. Layout & zoning (3-dimensional)

The zones are sized from the capacity analysis and laid out flow-through (receiving one side, shipping the other). Building ≈ 108 m × 70 m, providing ~14,074 pallet positions against the ~14,074 target.

| Zone | Area m² | % of building |
| --- | --- | --- |
| Receiving & inbound staging | 700 | 9% |
| Offices & amenities | 300 | 4% |
| Returns / VAS | 300 | 4% |
| Double-deep reserve (A/B) | 3,505 | 47% |
| Selective + cantilever (irregular/XLONG/C) | 728 | 10% |
| Forward-pick module (carton-flow/shelving) | 400 | 5% |
| Packing & consolidation | 700 | 9% |
| Outbound staging & shipping | 300 | 4% |
| Circulation / aisles | 560 | 7% |

**Total floor area 7,493 m² = 107% of a 7,000 m² envelope.**

At the detailed zoning level Option B lands slightly over 7,000 m² (107%) once full functional areas are included — tighter than the concept-level estimate.

Mitigations if 7,000 m² is the binding Assa-only envelope (A008): (a) raise the double-deep share or add an 8th rack level (12.2 m height allows it, subject to truck reach); (b) trim office/returns/staging; (c) switch reserve to VNA (Option C), which frees ~2,000 m² and gives real headroom.

Fig 5. Option B layout & zoning, to scale, with order flow (①inbound→⑥ship).

Fig 6. Option B 3D massing (isometric) — reserve racking ≈ 11.5 m within the 12.2 m limit.

Fig 7. Worker movement — zone picking keeps picker paths short and separated.

# 5. Capacity plan — labour & equipment

Workload drivers (measured): outbound ≈ 18 palletised shipments/day (rest parcel), inbound ≈ 32 pallets/day, forward-pick replen ≈ 22 pallets/day. Productivities are benchmark assumptions (A014).

| Process | Rate/h | FTE avg | FTE peak |
| --- | --- | --- | --- |
| Receiving (unload+check) | 25 | 0.17 | 0.48 |
| Putaway (reach) | 20 | 0.21 | 0.60 |
| Replenishment | 18 | 0.16 | 0.47 |
| Case picking | 80 | 0.87 | 1.59 |
| Each picking | 55 | 0.25 | 0.45 |
| Full-pallet picking | 22 | 0.10 | 0.18 |
| Packing/consolidation | 45 | 1.85 | 3.38 |
| Shipping/loading | 30 | 0.08 | 0.22 |

**Total direct labour ≈ 3.7 FTE average, 7.4 FTE at peak** (≈ 12 if each process is staffed separately; cross-training reduces this). Add supervision/admin and value-add separately.

Fig 8. Labour requirement by process (average vs peak).

Material-handling equipment (peak):

| Equipment | Peak equip-hours/day | Units |
| --- | --- | --- |
| Reach truck (putaway/replen/pallet pick) | 9.4 | 2 |
| Low-level order picker (case/each) | 15.3 | 3 |
| Pallet truck (receiving/shipping) | 5.3 | 1 |

Plus RF/voice terminals per picker, print-and-apply at pack, and a small spares allowance. Peak may need overtime or a partial second shift rather than extra permanent FTE.

# 6. Assumptions & open questions

Full register in data/assumptions.json. High-impact items for the future state:

| ID | Assumption / open question | Impact |
| --- | --- | --- |
| A001 | Max facility footprint | HIGH |
| A002 | Max clear/building height | HIGH |
| A003 | Analysis period is representative of normal operation | HIGH |
| A004 | One LODNUM (load) occupies one pallet position | HIGH |
| A005 | STAGED STATUS 'LATE' reflects staging vs the LATE SHIP window, i.e. a genuine on-time-staging miss | HIGH |
| A006 | No labour-productivity, MHE-fleet or cost data was provided | HIGH |
| A008 | The 7,000 m2 / 12.2 m envelope is the space available to DESIGN THE ASSA ABLOY operation within | HIGH |
| A010 | Design pallet-position target | HIGH |
| A014 | Labour productivity benchmarks (units/person-hour) | HIGH |