PROJECT OTM

Warehouse Design — Analysis & Policy Pre-selection

Assa Abloy operation · DHL Supply Chain, Bemmel (NL_0032)

| Field | Detail |
| --- | --- |
| Deliverable | 1 — Analysis of data, pre-selection & justification of policies |
| Due date | 16-09-2026 |
| Data period analysed | Q2 2025 (01 Apr – 30 Jun) |
| Site constraints | ≤ 7,000 m² footprint · ≤ 12.2 m height |
| Scope | Receiving · Storage · Picking · Shipping (end-to-end) |
| Author | ________________________ |
| Version | v2 — analysis, design options, policies & recommendation (updated with teacher feedback) |

This draft covers Deliverable 1 end-to-end: data analysis, storage-capacity feasibility, three design options, policy pre-selection, equipment, and a recommended concept with a decision matrix. All figures are measured from the provided Assa Abloy data or computed by the analysis scripts; nothing is fabricated, and open assumptions are listed in Section 13.

# Contents

(In Word: right-click → Update Field to build the table of contents.)

# 1. Introduction & scope

DHL Supply Chain previously ran warehouse operations for four clients in Tiel and recently relocated to a new site in Bemmel under significant time pressure. The Operations Manager considers the current (post-move) design sub-optimal. The objective of this project is to develop an alternative, more efficient and effective warehouse design for the Assa Abloy operation, based on a thorough analysis of the operational data.

The design covers four processes end-to-end — receiving, storage, picking and shipping — with all choices aligned across them. Facility architecture is out of scope; legal aspects (fire safety, compliance) are considered. The site envelope is a maximum footprint of 7,000 m² and a maximum height of 12.2 m.

# 2. Data sources & method

Two datasets from the Assa Abloy operation (warehouse NL_0032) form the primary source of truth:

| Dataset | Files | Grain | Volume |
| --- | --- | --- | --- |
| Storage / inventory | 3 snapshots (01-04/05/06-2025) | Inventory detail (DTLNUM) | 16,575–17,290 records |
| Outbound shipping & picking | 3 months (Apr–Jun 2025) | Order line | 39,069 lines · 8,781 orders |

Analysis is performed in Python (pandas). Every headline number is reproducible from the scripts in /analysis and cached in outputs/reports/phase1_metrics.json. Data-quality issues are flagged (Section 8), not silently corrected.

# 3. SKU profile

5,291 SKUs in stock across 11 product families.

3,115 SKUs have a case pack; 2,176 are each-only (≈41%) — confirming real each-pick demand.

Each size is small: median cube 0.0005 m³ (~500 cm³) — typical for locks, cylinders and fittings.

Units per pallet: median 1000 (mean skewed by high-count small parts).

Pallet weight: median 431 kg — realistic for a EURO pallet (see data-quality flag on sentinel maxima).

Pallet-type mix (SKU level): EURO 2,864, K3 2,040, XLONGPALLET 168 (long goods), plus K1/K2/K5/CONS.

Hazardous SKUs: 138 — require segregation/compliance consideration.

Fig 1. Pallet-weight distribution at SKU level (<p99).

# 4. Inventory & storage-position requirement

| Snapshot | Detail recs | Loads (positions) | Occupied locations | Active SKUs | Cube (m³) |
| --- | --- | --- | --- | --- | --- |
| 2025-04-01 | 16,575 | 10,176 | 7,715 | 5,164 | 3,209 |
| 2025-05-01 | 16,793 | 10,534 | 8,149 | 5,273 | 3,434 |
| 2025-06-01 | 17,290 | 11,015 | 8,512 | 5,291 | 3,679 |

Occupied locations (pallet positions) grew 7,715 → 8,512 over the quarter; loads grew +8.2% (10,176 → 11,015); cube +15%.

Peak pallet positions ≈ 8,512 occupied locations → the storage baseline. The 11,015 loads sit in those locations (some locations hold more than one load), so loads overstate the positions needed (A004).

Loads per SKU: mean 2.07, median 1, max 337; 72.7% of SKUs occupy a single load — a long slow-moving tail one pallet deep.

Eaches held roughly flat (~2.9M) while loads rose → stock is fragmenting into more, less-full locations: a current-design inefficiency to target.

Fig 2. Inventory footprint trend, Q2 2025 (loads and occupied locations).

Fig 2b. Stock ageing — 21% of loads have been in stock >1 year (dead-stock signal).

# 5. Order & picking profile and lead time

| Metric | Mean | Median | p95 | Max |
| --- | --- | --- | --- | --- |
| Lines per order | 4.45 | 1 | 19 | 191 |
| Units per line | 34.77 | 5 | 150 | 12020 |
| Units per order | 154.7 | 8 | 575 | 20480 |

8,781 orders / 39,069 lines; 50.5% single-line orders and 23.6% single-unit lines — a small-order-heavy profile.

Pick type is derived from what the client ordered (A022) — each line's quantity is compared with the SKU's case and pallet quantity: 75% each-pick lines, 24% case lines, 1% full-pallet lines (by units: 61% case, 26% each, 13% pallet).

Picking is each-dominant by line count (the labour driver) and case-dominant by volume. DHL's historical pick flags (case/list 60.8%, each 16.0%, 20.6% unflagged) reflect past practice and are not used for design.

Delivery lead time (order received → dispatched): average 78.3 h (~3.3 days), median 52.4 h (~2.2 days), p90 166.9 h. Right-skewed (mean > median), so the typical order is nearer the median; measured as TRAILER DISPATCED DATE − ORDER ARRIVE DATE.

Channel mix (order type): NLD 20,944, EDC 4,703, BEL 4,596, DIRECT 4,003, AMADE 2,216.

Geography: NLD 22,510, BEL 6,386, DEU 3,534, FRA 1,041, POL 911 — an NL hub serving Western Europe.

Fig 3. Pick type derived from the orders (line quantity vs case / pallet quantity).

# 6. Demand-based ABC classification

ABC is recomputed from actual outbound demand (picked quantity), not assumed. The distribution is steeper than the classic 80/20 rule:

| Top X% of SKUs | % of picked volume |
| --- | --- |
| 5% of SKUs | 58.9% |
| 10% of SKUs | 74.5% |
| 20% of SKUs | 88.1% |
| 30% of SKUs | 94.0% |
| 50% of SKUs | 98.5% |

A/B/C by demand volume (A ≤ 80% cumulative, B ≤ 95%, C remainder):

| Class | SKUs | % SKUs | % volume | % pick-lines |
| --- | --- | --- | --- | --- |
| A | 565 | 13.0% | 80.0% | 41.8% |
| B | 858 | 19.7% | 15.0% | 30.0% |
| C | 2,931 | 67.3% | 5.0% | 28.3% |

~13% of SKUs drive 80% of volume, yet C-class is 67% of SKUs and still 28% of picks → forward-pick / fast-lane slotting is high-value.

Demand-ABC agrees with the WMS classification on only 48.6% of SKUs → strong evidence the current slotting is misaligned with real demand (re-slot).

4,354 SKUs shipped in Q2; 855 shipped were not in the latest stock snapshot (transient/cross-dock — to confirm).

Fig 4. Demand Pareto — cumulative picked quantity vs ranked SKUs.

Fig 4b. Top-20 SKUs by demand — note several high-demand items still WMS-classed C.

# 7. Peak activity analysis

| Metric | Average / day | PEAK / day | Peak ÷ Avg | Peak date |
| --- | --- | --- | --- | --- |
| Order lines | 640 | 1171 | 1.83× | 2025-05-27 |
| Units | 22269 | 63469 | 2.85× | 2025-05-27 |
| Orders | 144 | 250 | 1.74× | 2025-04-29 |
| Shipments | 134 | 229 | 1.71× | 2025-04-29 |

Design to peak, not average. Units peak (2.85×) is far spikier than lines (1.83×), so sizing on lines alone understates pick/pack surges.

Weekly peak 3563 lines (week of 2025-06-01) vs ~2791 average.

The 08:00 intraday peak is caused by the order-release routine, not by customers (A023): 55% of orders arrive 15:00–17:00, are released 16:00–18:00 and are picked the next morning, so the backlog hits the floor at shift start (1.31× the average hour). Monday–Friday only, weekdays fairly even.

Fig 5. Daily shipped order-lines with average and peak.

Fig 6. Order arrival vs release vs picking by hour — the 08:00 peak is the release backlog.

# 8. Data-quality flags

Raised, not silently corrected:

PA WGT sentinel/impossible values — median 431 kg is fine, but p95 = 9999 kg and max = 60423 kg are impossible for one pallet (9,999 looks like a placeholder). To be cleaned before rack load-rating.

units_per_pallet extreme mean (15171 vs median 1000) — use robust statistics.

"Other" pick class = 20.6% of lines — no PALL/LIST/TROLLEY flag set. Resolved for the design: the pick type is derived from order quantities instead of these flags (A022).

Stock vs demand mismatch — 855 shipped-not-stocked; a further set of stocked-not-shipped SKUs are dead-stock candidates. To confirm handling (cross-dock vs obsolete).

# 9. Headline design requirements (input to design options)

Storage: ~8,512 pallet positions at peak → target ~9,500 (÷ 0.90 working utilisation); growth (+8%/quarter in loads) is absorbed by extra rack levels, not by an inflated count. Must fit the 12.2 m / 7,000 m² envelope (A008).

Long slow tail: 67.3% C-SKUs and 72.7% single-load SKUs → high-density/deep storage for the tail, fast forward-pick for A/B.

Picking is each-dominant by lines (75% each, 24% case, 1% full pallet — derived from the orders) and half of all orders are single-line → batch picking from an easy-reach fast-pick area is the key labour lever.

Re-slot to demand — only 48.6% ABC agreement today.

Capacity to peak — 1171 lines / 63469 units on the peak day; the 08:00 peak is a release effect that waves can level.

# 10. Storage capacity feasibility

Design target: peak 8,512 occupied locations ÷ 0.90 working utilisation ≈ 9,500 pallet positions (A010). This replaces the earlier 14,074 (loads × 1.15 buffer), which overstated the need by counting loads instead of positions. Measured pallet+load height is low (p90 116 cm), giving a 1.41 m level pitch and up to 8 levels under 12.2 m.

Bottom-up floor area needed to hold that target, by storage concept:

| Concept | MHE | Levels | Total area m² * | % of 7,000 m² | Fits? |
| --- | --- | --- | --- | --- | --- |
| Selective - wide aisle | Counterbalance FLT | 6 | 6,810 | 97% | TIGHT |
| Selective - narrow aisle | Reach truck | 7 | 5,040 | 72% | YES |
| Double-deep | Reach truck (deep) | 7 | 4,280 | 61% | YES |
| VNA | VNA turret truck | 8 | 3,390 | 48% | YES |
| Drive-in | Counterbalance FLT | 5 | 6,330 | 90% | TIGHT |

* includes a 35% allowance for receiving, staging, pick, pack, ship, offices and circulation.

At the corrected target several concepts fit: double-deep 61%, narrow-aisle selective 72%, VNA 48%. Wide-aisle (97%) and drive-in (90%) are tight.

→ Space alone no longer decides the concept. The choice is driven by travel and labour, slotting flexibility, growth headroom and cost — compared in the options and decision matrix below.

Fig 7. Floor area to hold the corrected position target, by concept, vs the 7,000 m² envelope.

# 11. Design options

Three end-to-end options (storage × picking × MHE × policies), each aligned across the four processes.

## Option A — Conventional+ (improve the current concept)

Storage: selective single-deep, narrow-aisle (reach), 7 levels; two-zone ABC. ≈72% of the envelope.

Picking: RF-directed discrete + simple batch; case pick from pick-face; pallet from reserve.

MHE: reach trucks, low-level order pickers, pallet trucks, RF scanners.

Verdict: cheapest and simplest, but the most aisles, travel and labour.

## Option B — Hybrid density + velocity slotting + zone/batch picking  (recommended)

Storage: double-deep reach racking for A/B reserve + selective single-deep for irregular SKUs + a forward-pick module (carton-flow / shelving) for fast case/each; cantilever for XLONG goods. ≈61% of the envelope for storage + working areas — room to grow.

Picking: zone + batch/wave picking (batches the 50% single-line orders); A-movers in a golden-zone forward pick; voice/RF; put-to-light consolidation at pack.

MHE: reach trucks with telescopic forks (Jungheinrich ETV 216i), pick trolleys for small orders and walkie pallet trucks (EJE 120i) for large orders, rider pallet trucks (ERE 225) at the docks, voice/RF, print-and-apply — sized in Deliverable 2.

Policies: velocity (demand-ABC) slotting, hybrid random-within-zone storage, min/max replenishment, FIFO via FIF DATE where relevant.

Verdict: fits with buffer, materially lower travel/labour than A, moderate CAPEX — strongest business case.

## Option C — High-density / semi-automated

Storage: VNA (man-up turret) for reserve + automated small-parts store (shuttle/AutoStore-style) for the large C each-pick tail. ≈48% of the envelope (densest).

Picking: goods-to-person / pick-to-light for smalls; VNA combined storage+pick; automated sortation.

MHE: VNA turret trucks + guidance, automation modules, minimal manual MHE.

Verdict: densest and lowest labour, but highest CAPEX and complexity; ROI depends on labour cost.

# 12. Decision matrix & recommendation

| Option | Weighted score (max 5) | Result |
| --- | --- | --- |
| B - Hybrid density | 3.75 | RECOMMENDED |
| C - Semi-automated | 3.70 | Strong alternative |
| A - Conventional+ | 3.05 | Baseline |

Weight-sensitivity (robustness):

| Option | Base | Cost-driven | Space-driven |
| --- | --- | --- | --- |
| B - Hybrid density | 3.75 | 3.65 | 4.75 |
| C - Semi-automated | 3.70 | 3.35 | 4.85 |
| A - Conventional+ | 3.05 | 3.35 | 3.55 |

Fig 8. Weighted decision-matrix scores.

Recommended: B - Hybrid density. It wins on base and cost-driven weightings; C overtakes only when space is the dominant criterion — likely if A008 resolves to a small Assa-only envelope.

Why (data): double-deep holds the corrected target in ≈61% of the envelope, leaving growth room with fewer aisles than selective racking; 75% of order lines are each-picks and 50% of orders single-line → batch/zone picking from a fast-pick area is the biggest labour lever; 48.6% ABC mismatch → velocity slotting is a cheap, high-value win; 73% single-load tail → dense reserve.

Conditions before locking: confirm the 7,000 m² scope (A008); obtain labour/equipment costs (A006) to confirm B vs C in the financial model; validate double-deep against the EURO/K3/XLONG pallet mix.

Recommended concept preview (detailed layout and 3D in Deliverable 2):

Fig 9. Recommended Option B — warehouse floor plan (rack-level, to scale).

# 13. Assumptions & open questions

Logged in data/assumptions.json. Items that materially affect the design:

| ID | Assumption / open question | Impact |
| --- | --- | --- |
| A001 | Max facility footprint | HIGH |
| A002 | Max clear/building height | HIGH |
| A003 | Analysis period is representative of normal operation | HIGH |
| A004 | Storage requirement is based on OCCUPIED LOCATIONS (pallet positions), not loads | HIGH |
| A005 | STAGED STATUS 'LATE' reflects staging vs the LATE SHIP window, i.e. a genuine on-time-staging miss | HIGH |
| A006 | No labour-productivity, MHE-fleet or cost data was provided | HIGH |
| A008 | The 7,000 m2 / 12.2 m envelope is the space available to DESIGN THE ASSA ABLOY operation within | HIGH |
| A010 | Design pallet-position target | HIGH |
| A013 | Functional (non-storage) areas sized from actual order volume | HIGH |
| A014 | Labour productivity benchmarks (units/person-hour) | HIGH |
| A017 | Peak-aware slotting: spread fast movers, duplicate extreme SKUs, balance picks/aisle on peak forecast | HIGH |
| A018 | Inbound arrives as 2 shipments per day, direct from Assa Abloy factories | HIGH |
| A019 | Part of inbound arrives LOOSE (floor-loaded sea containers) and is de-stuffed and palletised by hand | HIGH |
| A020 | Dock-door planning parameters | HIGH |
| A022 | Pick type is derived from what the client ordered, not from DHL's historical pick flags | HIGH |
| A024 | Picking is split by ORDER SIZE computed from SKU each-dimensions and weights | HIGH |
| A025 | Pick-time model parameters | HIGH |

Sources: project brief, the Assa Abloy data files, teacher/DHL feedback and external references (Gadeyne on peak slotting; Jungheinrich equipment data) — full list in documentation/sources.md.